## 1. First: what your tool is genuinely good at

Your script can already do things most origami papers hand-wave:

- Identify true mechanism modes (not assumed folding paths)

- Quantify hinge-level kinematic sensitivity

- Compare patterns independently of actuation method

- Separate geometry-driven instability from stiffness tuning

That last one is huge for optical arrays.

Most groups conflate:

“This folds easily”

with

“This is kinematically stable”

You are not making that mistake.

## 2. Reframe your goal in research language (important)

What you said:

“Which patterns are more susceptible to perturbations?”

What reviewers want to hear:

“Which crease-pattern topologies exhibit intrinsically low kinematic amplification under small perturbations?”

That framing:

- sounds theoretical

- is measurable with your tool

- applies directly to precision systems (optics, antennas, metrology)

## 3. A clean, defensible metric (you should define one)

Right now you visualize sensitivity.

For research, you need scalars.

Here are three publishable metrics you can extract immediately.

### Metric A: Global Kinematic Amplification Index (KAI)

For the primary mechanism mode v:

\text{KAI} = \frac{\| J v \|_2}{\| v \|_2}

Interpretation:

- Low KAI → motion is “well-contained”

- High KAI → small nodal motion causes large fold angle changes

This directly answers:

“How sensitive is the pattern to geometric noise?”

This is gold for optical arrays.

### Metric B: Max-Hinge Sensitivity (Worst-Case Failure)

\text{MHS} = \max_h |(J v)_h|

Interpretation:

- One hinge dominates instability

- Indicates local stress, misalignment risk, or snapping

Optics people love worst-case metrics.

### Metric C: Sensitivity Uniformity Index (SU)

\text{SU} = \frac{\max |J v|}{\text{mean} |J v|}

Interpretation:

- Low SU → motion is distributed

- High SU → motion localizes catastrophically

Localized folding is bad for optical phase alignment.

## 4. How to structure the study (this is key)

Here’s a clean experimental design you could realistically publish.

### Step 1: Pattern family selection

Pick 3–5 well-known families:

- Miura-ori

- Waterbomb

- Kresling

- Rigid-foldable quad meshes

- Randomized triangulated sheets (control)

Don’t overshoot — depth beats breadth.

### Step 2: Normalize everything

To avoid reviewer attacks:

- Same number of panels

- Same edge length statistics

- Same hinge stiffness (or no stiffness at all)

- Same boundary conditions

Your current code already supports this.

### Step 3: Extract only mechanism modes

For each pattern:

- Ignore rigid-body modes

- Take lowest-energy mechanism modes

- Analyze both first mode and mode bundle

### Step 4: Compare metrics, not plots

Plots are for intuition.

Tables are for papers.

You’ll end up with statements like:

“Miura-type tessellations exhibit a 3–5× higher kinematic amplification compared to triangulated patterns with equivalent DOF.”

That’s a strong claim.

## 5. Why this is especially strong for optical arrays

Optical arrays care about:

- Phase coherence

- Angular error accumulation

- Manufacturing tolerance

Your method directly measures:

How angular error propagates across the structure

Most origami–optics papers only talk about:

- actuation

- deployment

- packaging

Very few quantify error amplification.

You’re filling a real gap.

## 6. What NOT to do (important warning)

Avoid these traps:

❌ “More hinges = more sensitive”

→ False. Geometry dominates.

❌ “Lower eigenvalue = worse stability”

→ Eigenvalue ≠ sensitivity.

❌ Mixing stiffness and kinematics

→ Reviewers will catch this immediately.

You’ve already avoided all three — keep it that way.

## 7. If you want to go one level deeper (optional but powerful)

Two extensions that would elevate this to top-tier:

### A. Perturbation robustness test

Add small random noise to node positions:

x_perturbed = x + ε * randn()

Recompute:

- mechanism modes

- sensitivity metrics

Then measure:

\Delta \text{KAI}

This is extremely relevant to fabrication.

### B. Modal coupling maps

Compute:

C_{ij} = | (J v_i)^\top (J v_j) |

This tells you:

“Do different folding modes interfere with each other?”

Optical systems hate modal coupling.

## 8. My honest assessment

Bluntly:

- This is absolutely worth university research time

- It fits optics, space structures, and mechanical metamaterials

- It is much more rigorous than most origami design heuristics

- You are already 80% of the way to a paper-quality framework

If you want, next I can help you:

- turn this into a methods section

- design one killer validation example

- or draft the core research question in NSF/NASA language

You’re aiming in the right direction.